Smart Widget

GD32F350 USB-MIDI Firmware

Boot / DFU / App — Code Documentation

Version 1.0.0

Music Tribe · Embedded Systems

# Table of contents

# 1. Overview

The Smart Widget firmware is organized into three parts that form a complete firmware lifecycle: Boot (bootloader decision logic), DFU (in-application firmware update engine), and App (MIDI processing and routing). These three parts work together to ensure safe firmware updates and reliable MIDI operation.

MCU: GigaDevice GD32F350 (ARM Cortex-M4, 108 MHz, 64KB Flash, 16KB RAM)

USB: USBFS Full-Speed Device, Audio Class MIDI Streaming

MIDI: USB-MIDI 1.0 + DIN-5 UART at 31250 baud

Toolchain: Keil MDK, GD32F3x0 Firmware Library V2.5.0

## 1.1 Flash memory map

| Region | Address | Size | Part |
| --- | --- | --- | --- |
| Bootloader | 0x0800_0000 | 8 KB | Boot — separate Keil project, always resident |
| Application | 0x0800_2000 | 48 KB | App + DFU — main firmware, updated by DFU |
| Config | 0x0800_E000 | 4 KB | App — persistent settings (reserved) |
| DFU staging | 0x0800_F000 | 4 KB | Boot/DFU — magic flag for boot decision |

## 1.2 Three-part interaction

Boot reads the DFU flag and validates the application, then either jumps to App or enters DFU wait mode. App runs the MIDI system and, when the host initiates a firmware update, DFU receives firmware via SysEx, programs it into Flash, then triggers a reset. Boot re-runs, sees the updated app is valid, and jumps to the new firmware. The cycle is: Boot → App → DFU (update) → reset → Boot → App (new version).

# Part 1: Boot (bootloader)

boot_main.c is a standalone 8KB binary linked at 0x0800_0000 in a separate Keil project. It runs on every power-on or reset before the application. Its sole purpose is to decide: jump to the application, or stay in DFU wait mode.

## 2.1 Boot decision tree

Step 1: Read the 32-bit word at DFU_STAGING_ADDR (0x0800_F000).

- == 0xDEADBEEF — DFU upgrade was requested. Clear the flag (erase the staging page), then enter dfu_wait(). This happens when the App explicitly writes DFU_MAGIC_UPGRADE before resetting.

- ≠ 0xDEADBEEF — No upgrade request. Proceed to validate the application.

Step 2: Validate the application vector table at APP_ADDR (0x0800_2000).

- app_is_valid() — Reads the initial SP (first word) and reset handler (second word). SP must point into SRAM (0x2000_0000 to 0x2000_4000). Reset handler must point into the app Flash region (0x0800_2000 to 0x0800_E000).

- Valid — Disable interrupts, clear SysTick, clear NVIC pending, set SCB->VTOR = APP_ADDR, set MSP, jump to reset handler.

- Invalid — No valid firmware — enter dfu_wait().

## 2.2 DFU wait mode

When entering dfu_wait(), the bootloader blinks an LED (PB1) to indicate that no valid application exists. It periodically re-checks app_is_valid() in case firmware is flashed via SWD while the bootloader is running. A future version could implement USB-DFU class 1.1 here for recovery without a debugger.

## 2.3 jump_to_app() procedure

The jump sequence is critical for correct app startup:

| Step | Action | Reason |
| --- | --- | --- |
| 1 | __disable_irq() | Prevent interrupts during transition |
| 2 | SysTick->CTRL = 0 | Stop SysTick (bootloader may have enabled it) |
| 3 | NVIC->ICPR[i] = 0xFFFFFFFF | Clear all pending interrupts |
| 4 | SCB->VTOR = APP_ADDR | Relocate vector table to application |
| 5 | __set_MSP(app_sp) | Set stack pointer to app’s initial SP |
| 6 | app_reset() | Jump to app’s reset handler (address at APP_ADDR+4) |

## 2.4 Connection to DFU

The bootloader and DFU engine communicate through a single 32-bit flag at DFU_STAGING_ADDR:

| Flag value | Constant | Meaning |
| --- | --- | --- |
| 0xDEADBEEF | DFU_MAGIC_UPGRADE | App requests bootloader DFU mode (e.g. button held during reset) |
| 0x00000000 | DFU_MAGIC_NORMAL | DFU complete, boot normally (written by CMD_REBOOT) |
| 0xFFFFFFFF | (erased Flash) | Fresh/erased — treated same as NORMAL, validates app |

# Part 2: DFU (firmware update engine)

The DFU subsystem spans two layers: dfu_engine.c (Middleware) handles the protocol state machine and Flash programming, while on_sysex_rx() in main.c (App) dispatches incoming SysEx messages to the engine. The host sends firmware via USB-MIDI or UART SysEx; the device receives, verifies, and programs it without needing a debugger.

## 3.1 SysEx DFU protocol

All DFU messages follow a common format:

F0 <mfr> <dev> <model> <cmd> [payload...] F7

The application layer checks the manufacturer/device/model bytes before forwarding to dfu_process_sysex(). Messages not addressed to this device are ignored (but still forwarded to other ports by the router if MASK_SX rules allow).

## 3.2 Update sequence

### 3.2.1 CMD_START (0x01)

Host sends firmware total size (4 bytes big-endian) and expected CRC32 (4 bytes). The engine validates that size fits in APP region (48KB max), then initializes:

| Field | Initial value | Purpose |
| --- | --- | --- |
| state | DFU_RECEIVING | Accept data blocks |
| flash_addr | 0x0800_2000 | First app page |
| running_crc | 0 | Accumulate CRC32 |
| page_buf[1024] | 0xFF | Staging buffer for one Flash page |
| page_offset | 0 | Write position in page buffer |
| next_block | 0 | Expected block sequence number |

### 3.2.2 CMD_DATA (0x02)

Host sends firmware in sequential blocks (up to 128 bytes payload per SysEx message). Each block includes a 16-bit sequence number — out-of-order blocks trigger DFU_ERROR. Payload bytes are:

- page_buf[page_offset++] — Stored in the 1KB page staging buffer

- crc32(&byte, 1, running_crc) — CRC32 updated byte-by-byte

When page_offset reaches FMC_PAGE_SIZE (1024), flush_page() fires: erase the target Flash page at flash_addr, program page_buf word-by-word, advance flash_addr by 1024, reset page_offset to 0. Progress callback reports percentage.

### 3.2.3 CMD_VERIFY (0x03)

Flushes any remaining partial page (padded to 4-byte alignment with 0xFF). Compares running_crc with expected_crc. Match = DFU_COMPLETE (callback success=1), mismatch = DFU_ERROR (callback success=0).

### 3.2.4 CMD_REBOOT (0x04)

Writes DFU_MAGIC_NORMAL (0x00000000) to DFU_STAGING_ADDR, then calls NVIC_SystemReset(). The MCU resets and the bootloader takes over:

- Flag == NORMAL — Bootloader validates app → jump_to_app() with new firmware

- App corrupted — Bootloader’s app_is_valid() fails → enters dfu_wait()

## 3.3 DFU state machine

| State | Trigger | Next |
| --- | --- | --- |
| DFU_IDLE | CMD_START with valid size | DFU_RECEIVING |
| DFU_RECEIVING | CMD_DATA (correct sequence) | DFU_RECEIVING |
| DFU_RECEIVING | CMD_DATA (wrong sequence) | DFU_ERROR |
| DFU_RECEIVING | CMD_VERIFY + CRC match | DFU_COMPLETE |
| DFU_RECEIVING | CMD_VERIFY + CRC mismatch | DFU_ERROR |
| DFU_COMPLETE | CMD_REBOOT | Reset → Boot |

## 3.4 Flash protection

bsp_fmc_erase() and bsp_fmc_prog() enforce address bounds: only addresses in the range [APP_ADDR, CONFIG_ADDR) are accepted. Any attempt to write outside this range returns FMC_ERR_WRP. This protects the bootloader (below APP_ADDR) and the config/staging regions (at and above CONFIG_ADDR).

## 3.5 CRC engine

crc_engine.c provides CRC-16 (CCITT, polynomial 0x1021) and CRC-32 (Ethernet, polynomial 0xEDB88320). DFU uses CRC-32 for firmware integrity verification. Both functions support incremental computation with a running seed.

# Part 3: App (MIDI processing)

The application layer runs at 0x0800_2000 and provides bidirectional MIDI communication between USB and UART ports, with configurable routing rules. It also serves as the host for the DFU engine by dispatching SysEx messages.

## 4.1 Initialization sequence

| Step | Call | Purpose |
| --- | --- | --- |
| 1 | bsp_system_init() | Clock (48 MHz), SysTick (1ms), GPIO clocks, NVIC |
| 2 | router_init() | Clear routing tables, init parsers + SysEx contexts |
| 3 | dfu_init() | Clear DFU state machine |
| 4 | bsp_uart_midi_init() | USART0 + DMA for MIDI at 31250 baud |
| 5 | bsp_usb_init() | USBFS device, MIDI class, connect to bus |
| 6 | uart_bridge_init() | Register UART port with router |
| 7 | usb_bridge_init() | Register USB port with router, set ISR RX callback |
| 8 | router_set_sysex_cb() | Register on_sysex_rx() dispatcher |

## 4.2 Main loop

- router_poll_all() — Poll all ports, dual-feed bytes to midi_parse() and sx_feed(), route complete messages

- usb_bridge_drain_tx() — Pop channel/RT bytes from ring buffer, encode as USB-MIDI packets, send

- uart_bridge_drain_tx() — Pop bytes from ring buffer, send via DMA

## 4.3 MIDI RX path

### 4.3.1 USB RX

USBFS ISR pushes raw 4-byte USB-MIDI packets into g_usb_rx ring buffer. usb_port_poll() pops packets, decodes by CIN: 0x04/0x07 extract 3 SysEx bytes, 0x06 extracts 2, 0x05 extracts 1, others decoded via usb_midi_dec() for channel/system messages.

### 4.3.2 UART RX

DMA circular buffer at 31250 baud. uart_port_poll() reads available bytes from the DMA counter. Raw byte stream, no packet framing needed.

### 4.3.3 Router dual-feed

router_poll_all() feeds every received byte to two engines simultaneously:

- midi_parse() — State machine for channel/realtime messages. Produces midi_msg_t, forwarded via router_process() per routing rules.

- sx_feed() — Per-port SysEx assembler (512-byte buffer, 300ms timeout). On completion: notifies app callback + forwards to other ports per MASK_SX rules.

## 4.4 MIDI TX path

### 4.4.1 Channel / realtime

midi_api.c wrappers construct midi_msg_t and route through PORT_INT. Destination port’s send() is called: USB queues to ring buffer for drain, UART queues for DMA.

### 4.4.2 SysEx TX

When usb_port_send() detects data[0] == 0xF0, it bypasses the ring buffer and directly calls usb_midi_enc_sysex() to encode the complete F0..F7 into USB-MIDI packets (CIN 0x04/0x05/0x06/0x07) sent in 64-byte frames. UART SysEx TX is identical to channel TX (raw byte stream).

## 4.5 SysEx dispatch to DFU

on_sysex_rx() in main.c is the bridge between App and DFU. When a complete SysEx message arrives via the router callback:

- Check IDs — Verify mfr/dev/model bytes match this device

- Switch on cmd — Route DFU_CMD_START/DATA/VERIFY/REBOOT to dfu_process_sysex()

- Other commands — Future: config dump/load, identity request, preset management

## 4.6 Routing rules

| Field | Type | Description |
| --- | --- | --- |
| enabled | uint8_t | Master on/off switch |
| ch_mask | uint16_t | Bitmask of allowed channels (bit 0 = ch 1) |
| msg_mask | uint8_t | MASK_NOTE ¦ MASK_CC ¦ MASK_PC ¦ MASK_PB ¦ MASK_SX ¦ MASK_RT |

Default: USB ↔ UART bidirectional, all channels, MASK_ALL (0xFF).

# Appendix A: File inventory

| File | Part | Description |
| --- | --- | --- |
| boot_main.c | Boot | DFU flag check, app validation, VTOR jump, DFU wait |
| common_types.h | Shared | Flash map, pins, USB IDs, product info |
| main.c | App | Init, main loop, on_sysex_rx() dispatcher |
| midi_api.c/.h | App | Send wrappers: Note On/Off, CC, PC, PitchBend, etc. |
| usb_midi_bridge.c/.h | App | USB port: CIN decode, SysEx TX, ring buf drain |
| uart_midi_bridge.c/.h | App | UART port: DMA poll, raw send, drain |
| midi_parser.c/.h | DFU/App | Byte-by-byte MIDI parser with running status |
| midi_router.c/.h | DFU/App | Multi-port router + per-port SysEx assembly |
| sysex_assembler.c/.h | DFU/App | SysEx reassembly with timeout/overflow |
| usb_midi_codec.c/.h | DFU/App | USB-MIDI 1.0 packet encode/decode |
| ring_buffer.c/.h | DFU/App | Lock-free ring buffer (256 bytes) |
| crc_engine.c/.h | DFU | CRC-16 (CCITT) and CRC-32 (Ethernet) |
| dfu_engine.c/.h | DFU | SysEx DFU state machine, Flash programming |
| bsp_system.c/.h | Driver | Clock, SysTick, NVIC, delay, reset |
| bsp_usb_midi.c/.h | Driver | USBFS init, connect, send, RX callback |
| bsp_uart_midi.c/.h | Driver | USART0 + DMA for 31250 baud MIDI |
| bsp_fmc.c/.h | Driver | Flash erase/program with address protection |
| usb_midi_class.c/.h | Driver | USB class handler: EP setup, data callbacks |
| usb_midi_desc.c/.h | Driver | USB descriptors: device, config, strings |
| drv_usb_hw.c | Driver | USB clock, Timer2 delay |
| gd32f3x0_it.c/.h | Driver | ISR vectors: SysTick, USBFS, TIMER2, DMA |

# Appendix B: USB descriptor structure

| Descriptor | Bytes | Content |
| --- | --- | --- |
| Device descriptor | 18 | USB 2.0, VID 0x1209, PID 0x0001 |
| Configuration | 9 | 2 interfaces, bus-powered 100mA |
| Interface 0: Audio Control | 9+9 | AC header, points to MS interface |
| Interface 1: MIDI Streaming | 9+7 | MS header, 2 bulk endpoints |
| IN Jack Embedded (ID=1) | 6 | Receives from host via Bulk OUT |
| IN Jack External (ID=2) | 6 | Physical DIN-5 MIDI IN |
| OUT Jack Embedded (ID=3) | 9 | Sends to host via Bulk IN, src=Jack 2 |
| OUT Jack External (ID=4) | 9 | Physical DIN-5 MIDI OUT, src=Jack 1 |
| Bulk OUT EP 0x01 + CS-EP | 9+5 | Host → device, 64 bytes |
| Bulk IN EP 0x81 + CS-EP | 9+5 | Device → host, 64 bytes |

Total configuration descriptor: 101 bytes. Strings: MusicTrib (mfr), Smart Widget (product), 00000001 (serial).

# Appendix C: Driver API reference

## C.1 bsp_system

| Function | Description |
| --- | --- |
| bsp_system_init() | Clock 48 MHz, SysTick 1ms, peripheral clocks, NVIC |
| bsp_get_tick() | Return ms tick (volatile, from SysTick ISR) |
| bsp_delay_ms(ms) | Blocking delay |
| bsp_system_reset() | NVIC_SystemReset() |

## C.2 bsp_usb_midi

| Function | Description |
| --- | --- |
| bsp_usb_init() | GPIO, USB clock, NVIC, usbd_init, usbd_connect |
| bsp_usb_is_configured() | 1 if USB CONFIGURED state |
| bsp_usb_send(data, len) | Queue Bulk IN (max 64 bytes) |
| bsp_usb_tx_ready() | 1 if last IN transfer done |
| bsp_usb_set_rx_callback(cb) | Register Bulk OUT callback |

## C.3 bsp_uart_midi

| Function | Description |
| --- | --- |
| bsp_uart_midi_init() | USART0 31250 baud, DMA circular RX, DMA TX |
| bsp_uart_midi_send_dma(d, len) | Copy + DMA transmit |
| bsp_uart_midi_rx_available() | Unread byte count |
| bsp_uart_midi_rx_read() | Read one byte |

## C.4 bsp_fmc

| Function | Description |
| --- | --- |
| bsp_fmc_erase(addr) | Erase 1KB page (APP region only) |
| bsp_fmc_prog(addr, d, words) | Program words (bounds checked) |
| bsp_fmc_verify(addr, d, words) | Read-back verify |