Smart Widget

GD32F350 USB-MIDI Firmware

Code Documentation

Version 1.0.0

Music Tribe  ·  Embedded Systems

# Table of contents

# 1. Project overview

This document describes the firmware architecture and implementation of the Smart Widget, a GD32F350-based USB-MIDI device. The firmware provides bidirectional MIDI communication between a USB host and a DIN-5 MIDI port, with support for SysEx message processing and in-application firmware updates (DFU).

MCU: GigaDevice GD32F350 (ARM Cortex-M4, 108 MHz, 64KB Flash, 16KB RAM)

USB: USBFS Full-Speed Device, Audio Class MIDI Streaming

MIDI: USB-MIDI 1.0 + DIN-5 UART at 31250 baud

Toolchain: Keil MDK (ARM Compiler), GD32F3x0 Firmware Library V2.5.0

VID/PID: 0x1209 / 0x0001 (pid.codes open-source)

# 2. Architecture

The firmware follows a four-layer architecture. Each layer only depends on layers below it, ensuring clean separation of concerns.

| Layer | Role | Dependencies |
| --- | --- | --- |
| App | Application logic, MIDI API, bridge modules, main loop | Middleware, Driver |
| Middleware | MIDI parser, router, SysEx assembler, USB-MIDI codec, CRC, DFU engine | Driver (bsp_system only) |
| Driver / BSP | Hardware abstraction: USB, UART, Flash, system clock, interrupts | GD32 Firmware Library |
| Bootloader | Standalone 8KB binary at 0x0800_0000, validates app and jumps | None (self-contained) |

## 2.1 File inventory

| File | Layer | Description |
| --- | --- | --- |
| common_types.h | Shared | Flash map, pin defs, USB IDs, product info, ring size |
| main.c | App | Init sequence, main loop, SysEx dispatcher |
| midi_api.c/.h | App | Convenience wrappers for sending MIDI (Note, CC, PC, etc.) |
| usb_midi_bridge.c/.h | App | USB port: ISR callback, CIN decode, SysEx TX, ring buf drain |
| uart_midi_bridge.c/.h | App | UART port: DMA poll, raw send, ring buf drain |
| midi_parser.c/.h | Middleware | Byte-by-byte MIDI parser with running status |
| midi_router.c/.h | Middleware | Multi-port router with channel/type filters + SysEx assembly |
| sysex_assembler.c/.h | Middleware | SysEx reassembly with timeout and overflow protection |
| usb_midi_codec.c/.h | Middleware | USB-MIDI 1.0 event packet encoder/decoder |
| ring_buffer.c/.h | Middleware | Lock-free single-producer/single-consumer ring buffer |
| crc_engine.c/.h | Middleware | CRC-16 (CCITT) and CRC-32 (Ethernet) bit-by-bit |
| dfu_engine.c/.h | Middleware | SysEx-based DFU: receive, verify CRC32, program Flash |
| bsp_system.c/.h | Driver | Clock config, SysTick, NVIC, delay, reset |
| bsp_usb_midi.c/.h | Driver | USBFS init, connect, send, RX callback registration |
| bsp_uart_midi.c/.h | Driver | USART0 + DMA TX/RX for MIDI at 31250 baud |
| bsp_fmc.c/.h | Driver | Flash erase, program, verify with address protection |
| usb_midi_class.c/.h | Driver | USB class handler: EP setup, IN/OUT data callbacks |
| usb_midi_desc.c/.h | Driver | USB descriptors: device, config, AC/MS interfaces, strings |
| drv_usb_hw.c | Driver | USB clock config, Timer2-based delay for USB library |
| gd32f3x0_it.c/.h | Driver | Interrupt vectors: SysTick, USBFS, TIMER2, DMA, USART |
| boot_main.c | Bootloader | DFU flag check, app SP validation, VTOR set, jump to app |

# 3. MIDI message processing

## 3.1 RX path (host to device)

Incoming MIDI data arrives on two physical ports and converges in the router.

### 3.1.1 USB RX

The USBFS interrupt triggers when the host sends a Bulk OUT packet (up to 64 bytes, containing 1–16 USB-MIDI event packets of 4 bytes each). The class handler calls the RX callback registered by usb_midi_bridge, which pushes the raw bytes into the g_usb_rx ring buffer.

When router_poll_all() calls usb_port_poll(), it pops 4-byte packets from the ring buffer and decodes them by CIN (Code Index Number):

| CIN | Meaning | Bytes extracted |
| --- | --- | --- |
| 0x04 | SysEx start / continue | 3 data bytes (MIDI0, MIDI1, MIDI2) |
| 0x05 | SysEx end, 1 byte | 1 byte |
| 0x06 | SysEx end, 2 bytes | 2 bytes |
| 0x07 | SysEx end, 3 bytes | 3 bytes |
| 0x08–0x0E | Channel / system message | Decoded via usb_midi_dec() |

### 3.1.2 UART RX

USART0 receives raw MIDI bytes at 31250 baud via DMA in circular mode into a 256-byte buffer. uart_port_poll() reads available bytes by comparing the DMA transfer counter with the software tail pointer. No packet framing is needed since UART MIDI is a raw byte stream.

### 3.1.3 Router dual-feed

router_poll_all() iterates over all registered ports, calls each port’s poll function, then feeds every received byte to two engines simultaneously:

- midi_parse() — State machine that assembles channel messages (Note On/Off, CC, PC, Pitch Bend) and realtime messages (Clock, Start, Stop). Produces a midi_msg_t when complete, which router_process() forwards to other ports per routing rules.

- sx_feed() — Per-port SysEx assembler that collects bytes between F0 and F7 into a 512-byte buffer. Includes 300ms timeout and overflow protection. When complete, the app callback is invoked and the message is forwarded to other ports if MASK_SX is enabled in the routing rules.

## 3.2 TX path (device to host)

### 3.2.1 Channel / realtime messages

Application code calls midi_api functions (e.g., midi_send_note_on) which construct a midi_msg_t and pass it to router_process(PORT_INT, &msg). The router matches against rules and calls the destination port’s send() function.

For USB: raw MIDI bytes are pushed into g_usb_tx. The main loop calls usb_bridge_drain_tx(), which pops bytes, re-parses them through midi_parse() to recover message boundaries, encodes each into a 4-byte USB-MIDI packet via usb_midi_enc(), and sends in 64-byte Bulk IN frames.

For UART: raw bytes are pushed into a ring buffer. uart_bridge_drain_tx() pops them into a linear buffer and sends via DMA.

### 3.2.2 SysEx messages

When the router forwards a complete SysEx message to the USB port, usb_port_send() detects data[0] == 0xF0 and takes a direct path: usb_midi_enc_sysex() encodes the entire F0..F7 buffer into multiple USB-MIDI packets (CIN 0x04 for continuation, 0x05/0x06/0x07 for the final packet), which are sent in 64-byte frames. This bypasses the ring buffer and drain, avoiding the problem of midi_parse() dropping SysEx data bytes.

For UART, SysEx and channel messages use the same send() path since UART is a raw byte stream.

## 3.3 Routing rules

Each source–destination port pair has an independent route_rule_t with three filter dimensions:

| Field | Type | Description |
| --- | --- | --- |
| enabled | uint8_t | Master on/off switch for this route |
| ch_mask | uint16_t | Bitmask of allowed MIDI channels (bit 0 = ch 1, bit 15 = ch 16) |
| msg_mask | uint8_t | Bitmask of allowed message types: MASK_NOTE, MASK_CC, MASK_PC, MASK_PB, MASK_SX, MASK_RT |

Default configuration: USB ↔ UART bidirectional, all channels, all message types (MASK_ALL = 0xFF).

# 4. MIDI parser

midi_parser.c implements a byte-by-byte state machine that handles running status, interleaved realtime messages, and SysEx detection.

## 4.1 State machine

| State | Trigger | Action |
| --- | --- | --- |
| PS_IDLE | Status byte (0x80–0xEF) | Store status, enter PS_D1 |
| PS_IDLE | Data byte + running status | Reuse last status, fill data[0] |
| PS_D1 | Data byte | Fill data[0], check if 1-byte or 2-byte msg |
| PS_D2 | Data byte | Fill data[1], emit complete midi_msg_t |
| PS_SYSEX | 0xF0 received | Enter SysEx passthrough (handled by sx_feed) |
| Any | Realtime (0xF8–0xFF) | Emit immediately, state unchanged |

## 4.2 API

| Function | Description |
| --- | --- |
| midi_parser_init(p) | Reset parser state to PS_IDLE, clear running status |
| midi_parse(p, byte, out) | Feed one byte; returns 1 if complete midi_msg_t produced |
| midi_data_len(status) | Returns expected data byte count for a status byte (0, 1, or 2) |
| midi_encode_msg(m, buf, max) | Serialize midi_msg_t to raw bytes (1–3 bytes) |

# 5. USB-MIDI codec

usb_midi_codec.c handles the translation between raw MIDI bytes and USB-MIDI 1.0 event packets. Each packet is 4 bytes: [CIN|Cable] [MIDI0] [MIDI1] [MIDI2].

## 5.1 API

| Function | Description |
| --- | --- |
| usb_midi_enc(msg, cable, pkt) | Encode a midi_msg_t into a single USB-MIDI event packet |
| usb_midi_dec(pkt, msg) | Decode a USB-MIDI event packet into a midi_msg_t |
| usb_midi_enc_sysex(sx, len, cable, pkts, max) | Encode a complete SysEx buffer into multiple USB-MIDI packets |

## 5.2 SysEx encoding

usb_midi_enc_sysex() splits the F0..F7 byte stream into 3-byte groups. Each group becomes a USB-MIDI packet with CIN 0x04 (start/continue). The final packet uses CIN 0x05 (1 byte remaining), 0x06 (2 bytes), or 0x07 (3 bytes ending with F7). Unused bytes in the final packet are zero-padded.

# 6. SysEx assembler

sysex_assembler.c reassembles variable-length SysEx messages (F0..F7) from an incoming byte stream. Used per-port inside the router to handle multi-packet SysEx from both USB and UART.

## 6.1 State machine

| State | Description |
| --- | --- |
| SX_IDLE | Waiting for F0 start byte |
| SX_RECEIVING | Collecting data bytes into buf[], refreshing timestamp on each byte |
| SX_COMPLETE | F7 received, complete message in buf[0..len-1] |
| SX_ERR_OVERFLOW | Message exceeded SX_MAX_LEN (512 bytes) |
| SX_ERR_TIMEOUT | No byte received within SX_TIMEOUT (300ms) |

## 6.2 Configuration

| Constant | Value | Purpose |
| --- | --- | --- |
| SX_MAX_LEN | 512 | Maximum SysEx message size in bytes |
| SX_TIMEOUT | 300 | Timeout in milliseconds between consecutive bytes |

Realtime bytes (0xF8–0xFF) appearing inside SysEx are ignored per the MIDI specification. Non-realtime status bytes (0x80–0xF7, except F7) abort the current SysEx and return to SX_IDLE.

# 7. DFU engine

dfu_engine.c implements in-application firmware update via SysEx. The host sends firmware data in a four-phase protocol; the device receives, verifies (CRC32), and programs it into Flash.

## 7.1 SysEx DFU protocol

All DFU SysEx messages follow the format:

F0 <mfr> <dev> <model> <cmd> [payload...] F7

The application-layer dispatcher in main.c checks the manufacturer/device/model bytes before forwarding to dfu_process_sysex().

## 7.2 Command sequence

### Phase 1: DFU_CMD_START (0x01)

Host sends firmware total size (4 bytes, big-endian) and expected CRC32 (4 bytes). The engine validates that the size fits in the application region (APP_ADDR to CONFIG_ADDR = 48KB), then initializes: flash_addr = 0x0800_2000, running_crc = 0, page_buf filled with 0xFF, state = DFU_RECEIVING.

### Phase 2: DFU_CMD_DATA (0x02)

Host sends firmware in sequential blocks (128-byte payload per SysEx message). Each block carries a 16-bit sequence number for ordering verification. Payload bytes are accumulated in a 1KB page buffer while updating the running CRC32. When the page buffer fills (1024 bytes), it triggers a flash page cycle: erase the target page, then program word-by-word. Progress is reported via callback.

### Phase 3: DFU_CMD_VERIFY (0x03)

Host sends the verify command. Any remaining partial page is padded to 4-byte alignment with 0xFF and flushed to Flash. The engine compares running_crc against expected_crc. Match = DFU_COMPLETE, mismatch = DFU_ERROR. Result reported via callback.

### Phase 4: DFU_CMD_REBOOT (0x04)

Writes DFU_MAGIC_NORMAL (0x00000000) to DFU_STAGING_ADDR (0x0800_F000) and triggers NVIC_SystemReset(). The bootloader reads this flag on next boot to decide whether to enter DFU mode or jump to the application.

## 7.3 DFU state machine

| State | Description |
| --- | --- |
| DFU_IDLE | No update in progress |
| DFU_RECEIVING | Accepting data blocks, programming Flash page-by-page |
| DFU_COMPLETE | CRC32 verified, firmware is valid |
| DFU_ERROR | Size overflow, sequence error, or CRC mismatch |

# 8. Flash memory map

| Region | Address | Size | Description |
| --- | --- | --- | --- |
| Bootloader | 0x0800_0000 | 8 KB | Standalone boot binary, checks DFU flag and app validity |
| Application | 0x0800_2000 | 48 KB | Main firmware, VTOR offset = 0x2000 |
| Config | 0x0800_E000 | 4 KB | Persistent configuration storage (reserved) |
| DFU staging | 0x0800_F000 | 4 KB | DFU magic flag: 0xDEADBEEF = upgrade, 0x00000000 = normal |

Flash page size on GD32F350 is 1024 bytes. The bsp_fmc driver enforces address bounds: erase and program operations are rejected if the target address falls outside the APP_ADDR to CONFIG_ADDR range, protecting the bootloader and config regions.

# 9. Bootloader

boot_main.c is a separate Keil project linked at 0x0800_0000 (8KB ROM). It executes on every power-on/reset before the application.

## 9.1 Boot sequence

Step 1: Read the 32-bit word at DFU_STAGING_ADDR. If it equals DFU_MAGIC_UPGRADE (0xDEADBEEF), clear the flag and enter DFU wait mode (bootloader-level DFU, not implemented in this version).

Step 2: Read the initial stack pointer value at APP_ADDR (first word of the application vector table). If it points to valid RAM (0x2000_0000 to 0x2000_4000), the application is considered valid.

Step 3: Set SCB->VTOR = APP_ADDR to relocate the vector table, load the app’s SP, and jump to the app’s reset handler (second word at APP_ADDR + 4).

If the application is invalid, the bootloader remains in DFU wait mode.

# 10. Driver layer reference

## 10.1 bsp_system

| Function | Description |
| --- | --- |
| bsp_system_init() | Configure clock (48 MHz), SysTick (1ms), enable peripheral clocks |
| bsp_nvic_config() | Set NVIC priority group PRE4_SUB0, enable all used IRQs |
| bsp_get_tick() | Return millisecond tick counter (volatile, incremented by SysTick ISR) |
| bsp_delay_ms(ms) | Blocking delay using tick counter |
| bsp_system_reset() | Trigger NVIC_SystemReset() |

## 10.2 bsp_usb_midi

| Function | Description |
| --- | --- |
| bsp_usb_init() | Init GPIO (PA11/PA12), USB clock, NVIC, usbd_init, usbd_connect |
| bsp_usb_is_configured() | Returns 1 if USB device is in CONFIGURED state |
| bsp_usb_send(data, len) | Queue data for Bulk IN transfer (max 64 bytes) |
| bsp_usb_tx_ready() | Returns 1 if last IN transfer completed |
| bsp_usb_set_rx_callback(cb) | Register callback for Bulk OUT data (ISR context) |

## 10.3 bsp_uart_midi

| Function | Description |
| --- | --- |
| bsp_uart_midi_init() | Configure USART0 at 31250 baud, DMA circular RX, DMA normal TX |
| bsp_uart_midi_send_dma(data, len) | Copy data to TX buffer and start DMA transfer |
| bsp_uart_midi_tx_busy() | Returns 1 if DMA TX transfer is in progress |
| bsp_uart_midi_rx_available() | Returns number of unread bytes in DMA circular RX buffer |
| bsp_uart_midi_rx_read() | Read and consume one byte from RX buffer |

## 10.4 bsp_fmc

| Function | Description |
| --- | --- |
| bsp_fmc_erase(addr) | Erase one 1KB Flash page (address must be in APP region) |
| bsp_fmc_prog(addr, data, words) | Program word array to Flash (address bounds checked) |
| bsp_fmc_verify(addr, data, words) | Read-back verify programmed data |

# 11. USB descriptor structure

The device enumerates as a USB Audio Class device with one MIDI Streaming interface. The descriptor tree is:

| Descriptor | Size | Content |
| --- | --- | --- |
| Device descriptor | 18 B | USB 2.0, VID 0x1209, PID 0x0001, per-interface class |
| Configuration | 9 B | 2 interfaces (AC + MS), bus-powered 100mA |
| Interface 0: Audio Control | 9 B | No endpoints, AC header points to interface 1 |
| AC Header | 9 B | bcdADC 1.00, 1 streaming interface |
| Interface 1: MIDI Streaming | 9 B | 2 endpoints (Bulk IN + OUT) |
| MS Header | 7 B | bcdMSC 1.00, wTotalLength 65 |
| MIDI IN Jack (Embedded, ID=1) | 6 B | Receives data from host via Bulk OUT |
| MIDI IN Jack (External, ID=2) | 6 B | Represents physical DIN-5 MIDI IN |
| MIDI OUT Jack (Embedded, ID=3) | 9 B | Sends data to host via Bulk IN, source = Jack 2 |
| MIDI OUT Jack (External, ID=4) | 9 B | Represents physical DIN-5 MIDI OUT, source = Jack 1 |
| Bulk OUT EP (0x01) | 9+5 B | Host to device, 64 bytes, associated with Jack 1 |
| Bulk IN EP (0x81) | 9+5 B | Device to host, 64 bytes, associated with Jack 3 |

Total configuration descriptor length: 101 bytes. String descriptors provide manufacturer (MusicTrib), product (Smart Widget), and serial number (00000001).

# 12. Ring buffer

ring_buffer.c provides a power-of-2 ring buffer (256 bytes default) for ISR-safe byte queuing. Uses volatile head/tail indices with mask-based wrapping. Single-producer/single-consumer safe without locks.

## 12.1 API

| Function | Description |
| --- | --- |
| rb_init(rb) | Reset head and tail to 0 |
| rb_push(rb, byte) | Push one byte; returns 0 if full |
| rb_pop(rb, *byte) | Pop one byte; returns 0 if empty |
| rb_push_blk(rb, data, len) | Push block; returns count actually pushed |
| rb_pop_blk(rb, data, max) | Pop block; returns count actually popped |
| rb_avail(rb) | Returns number of bytes available to read |
| rb_flush(rb) | Discard all data (set tail = head) |

# 13. Initialization and main loop

## 13.1 Initialization order

The init sequence in main() is designed so that each component’s dependencies are ready before it initializes:

| Step | Call | Purpose |
| --- | --- | --- |
| 1 | bsp_system_init() | Clock, SysTick, GPIO clocks, NVIC priorities |
| 2 | router_init() | Clear routing tables, init parsers and SysEx contexts |
| 3 | dfu_init() | Clear DFU state machine context |
| 4 | bsp_uart_midi_init() | Configure USART0 + DMA for MIDI |
| 5 | bsp_usb_init() | Configure USBFS, register descriptors, connect to bus |
| 6 | uart_bridge_init() | Register UART port with router |
| 7 | usb_bridge_init() | Register USB port with router, set ISR RX callback |
| 8 | router_set_sysex_cb() | Register application SysEx dispatcher |

## 13.2 Main loop

The main loop runs three functions on every iteration:

- router_poll_all() — Polls all registered ports for incoming bytes, feeds them to both the channel parser and SysEx assembler, and routes complete messages.

- usb_bridge_drain_tx() — Pops queued channel/RT bytes from g_usb_tx, re-encodes as USB-MIDI packets, and sends to the host.

- uart_bridge_drain_tx() — Pops queued bytes from the UART TX ring buffer and sends via DMA.